# MUSHROOM FARMING IoT PROJECT

Complete Technical Documentation

# TABLE OF CONTENTS

1. Project Overview

2. Hardware Requirements

3. Pin Diagram & Connections

4. Software Architecture

5. Features & Functionality

6. Sensor Details

7. Menu System

8. EEPROM Memory Layout

9. API Endpoints

10. Code File Structure

11. Usage Instructions

12. Wireless OTA Updates

13. Troubleshooting

# 1. PROJECT OVERVIEW

The Mushroom Farming IoT Project is an automated system designed to control and monitor temperature, humidity, and CO2 levels in mushroom farms. It allows users to monitor and control their farms remotely using mobile devices and web portals.

## Key Objectives:

Optimize growth conditions for mushroom cultivation

Minimize crop losses caused by human error

Address agricultural challenges through automation

Provide real-time monitoring and remote control capabilities

Enable data logging and analysis for better farm management

## System Capabilities:

Real-time monitoring of CO2, temperature, and humidity levels

Automated relay control for maintaining optimal conditions

WiFi connectivity for remote access and data transmission

LCD display for local monitoring and menu navigation

Joystick-based user interface for easy configuration

Over-the-air (OTA) firmware updates

Device key authentication for secure access

# 2. HARDWARE REQUIREMENTS

| Component | Model/Specification | Purpose |
| --- | --- | --- |
| Microcontroller | ESP32 WROOM | Main processing unit with WiFi |
| CO2 Sensor | SCD41 | Measures CO2, temperature, and humidity |
| Bag Sensors | DS18B20 | Temperature sensors for mushroom bags |
| Room Monitor | DHT11 | Outside room temperature and humidity |
| Display | LCD I2C 20x4 | Local display and menu interface |
| Input Device | Joystick Module | Menu navigation and input |
| Power Supply | DC 5V, 2A Adapter | System power supply |

Important Note: For setup inside rooms, it is crucial to heat seal the wiring to prevent any shorting due to high moisture. Ensure proper heat sealing to maintain system integrity.

# 3. PIN DIAGRAM & CONNECTIONS

## 3.1 I2C Devices (Shared Bus)

| Device | SDA | SCL | I2C Address |
| --- | --- | --- | --- |
| LCD Display | GPIO 21 | GPIO 22 | 0x27 |
| SCD41 CO2 Sensor | GPIO 21 | GPIO 22 | Default |

## 3.2 Digital Sensors

| Sensor | Pin | Protocol |
| --- | --- | --- |
| DHT11 (Outside Room) | GPIO 5 | Digital |
| DS18B20 Bus 1 (Bags) | GPIO 0 | OneWire |
| DS18B20 Bus 2 (Bags) | GPIO 17 | OneWire |

## 3.3 Joystick Input

| Function | Pin | Type |
| --- | --- | --- |
| Joystick X-Axis | GPIO 32 | Analog Input |
| Joystick Y-Axis | GPIO 33 | Analog Input |
| Joystick Button | GPIO 26 | Digital Input (Pull-up) |

## 3.4 Relay Outputs

| Relay | Pin | Purpose |
| --- | --- | --- |
| HUMIDITY_RELAY_1 | GPIO 23 | Humidity Control |
| TEMP_RELAY_2 | GPIO 4 | AC/Temperature Control |
| CO2_RELAY_3 | GPIO 16 | CO2 Control |

## 3.5 Complete Pin Reference

| GPIO Pin | Function | Direction | Notes |
| --- | --- | --- | --- |
| 0 | DS18B20 Bus 1 | Input | OneWire protocol |
| 4 | TEMP_RELAY_2 | Output | AC Control |
| 5 | DHT11 | Input | DHT11 Sensor |
| 16 | CO2_RELAY_3 | Output | CO2 Control |
| 17 | DS18B20 Bus 2 | Input | OneWire protocol |
| 21 | I2C SDA | Bidirectional | LCD & SCD41 |
| 22 | I2C SCL | Output | LCD & SCD41 |
| 23 | HUMIDITY_RELAY_1 | Output | Humidity Control |
| 26 | Joystick Button | Input | Internal Pull-up |
| 32 | Joystick X | Input | Analog |
| 33 | Joystick Y | Input | Analog |

# 4. SOFTWARE ARCHITECTURE

## 4.1 Core Components

The software is built on the Arduino framework for ESP32, utilizing multiple libraries for sensor interfacing, WiFi connectivity, and HTTP communication.

### Libraries Used:

SensirionI2CScd4x - SCD41 CO2 sensor interface

DHT-sensor-library - DHT11 temperature/humidity sensor

DallasTemperature - DS18B20 temperature sensors

OneWire - OneWire protocol implementation

LiquidCrystal_I2C - LCD display control

WiFi & HTTPClient - Network communication

ESPAsyncWebServer - Async web server for OTA

AsyncElegantOTA - Over-the-air firmware updates

ESPAsyncWiFiManager - WiFi configuration portal

ArduinoJson - JSON parsing and generation

EEPROM - Non-volatile storage

## 4.2 Program Flow

1. Setup Phase:

- Initialize Serial communication

- Initialize LCD display

- Connect to WiFi (or create AP for configuration)

- Initialize all sensors (SCD41, DHT11, DS18B20)

- Initialize joystick and relay pins

- Load settings from EEPROM

- Authenticate device key with server

- Display welcome screen

2. Main Loop:

- Check for joystick button press (menu trigger)

- Authenticate device key every 30 minutes

- Read sensor data continuously

- Send data to server every 5 minutes

- Update relay states based on thresholds

- Commit EEPROM changes

# 5. FEATURES & FUNCTIONALITY

## 5.1 Sensor Monitoring

The system continuously monitors multiple environmental parameters:

CO2 Levels: Measured by SCD41 sensor (0-40000 ppm range)

Temperature: Monitored by SCD41 (room), DS18B20 (bags), and DHT11 (outside)

Humidity: Tracked by SCD41 (room) and DHT11 (outside)

Up to 8 bag sensors can be connected (4 per OneWire bus)

## 5.2 Automated Relay Control

Three relays are controlled automatically based on configurable thresholds:

CO2 Relay: Activates when CO2 < CO2MinValue, deactivates when CO2 > CO2MinValue + 100

Humidity Relay: Activates when humidity >= humidityMin, deactivates when humidity < humidityMin - 2.5

Temperature Relay (AC): Activates when temperature <= tempMinValue, deactivates when temperature > tempMinValue + 1

Default Threshold Values:

CO2 Minimum: 1200 ppm

Temperature Minimum: 16°C

Humidity Minimum: 90%

## 5.3 Data Transmission

Sensor data is transmitted to the cloud server every 5 minutes via HTTP POST requests. Three separate APIs are used for different sensor types:

## 5.4 WiFi Management

Auto-connect to saved WiFi networks

WiFi configuration portal (AP mode) when no saved network available

Automatic reconnection on disconnect

WiFi settings stored in ESP32 flash

## 5.5 Security Features

12-character device key authentication

Device key validated every 30 minutes

Subscription-based model - device stops if subscription expires

Factory reset requires secure code (AB1234)

# 6. SENSOR DETAILS

## 6.1 SCD41 CO2 Sensor

Interface: I2C (shared with LCD)

Measurements: CO2 (ppm), Temperature (°C), Humidity (%)

Update Rate: Periodic measurement mode

Calibration: Automatic baseline calibration

Data Ready: Polling-based data ready flag checking

## 6.2 DHT11 Sensor

Pin: GPIO 5

Measurements: Temperature (°C), Humidity (%)

Type: Digital sensor

Purpose: Outside room monitoring

Heat Index: Calculated for comfort monitoring

## 6.3 DS18B20 Bag Sensors

Bus 1: GPIO 0

Bus 2: GPIO 17

Protocol: OneWire

Max Sensors: 4 per bus (8 total)

Measurement: Temperature only (°C)

Purpose: Individual mushroom bag monitoring

# 7. MENU SYSTEM

The system features an interactive menu system controlled via joystick. Press the joystick button to enter the menu from the main screen.

## 7.1 Menu Options

| Menu Option | Description |
| --- | --- |
| Change Relay Value | Modify CO2, humidity, and temperature thresholds |
| Reset to Default | Restore default threshold values (CO2: 1000, Temp: 16°C, Hum: 80%) |
| Factory Reset | Clear all settings and EEPROM data (requires reset code) |
| Change WiFi | Reset WiFi settings and restart configuration portal |
| Display Bag Reading | Show detailed temperature readings from all bag sensors |
| Restart | Reboot the ESP32 device |

## 7.2 Virtual Keyboard

The system includes a 3-mode virtual keyboard for text input:

Small Letters (a-z): Default mode for general input

Capital Letters (A-Z): For uppercase input

Special Characters: Numbers and symbols for values

Keyboard Controls:

Move joystick: Navigate through keys

Press button: Select character

# key: Switch to special characters

A key: Switch to capital letters

a key: Switch to small letters

ENT: Confirm input

BCK: Backspace/delete

Menu timeout: 2 minutes of inactivity

# 8. EEPROM MEMORY LAYOUT

The system uses EEPROM to store configuration and state data persistently. Total EEPROM size allocated: 32 bytes

| Address | Size | Variable | Description |
| --- | --- | --- | --- |
| 0 | 1 byte | ADDR_CO2_RELAY_STATUS | CO2 relay state (0/1) |
| 1 | 1 byte | ADDR_HUM_RELAY_STATUS | Humidity relay state (0/1) |
| 2 | 1 byte | ADDR_AC_RELAY_STATUS | AC relay state (0/1) |
| 3-4 | 2 bytes | ADDR_MIN_VAL_CO2 | CO2 minimum threshold |
| 5-8 | 4 bytes | ADDR_MIN_VAL_TEMP | Temperature minimum threshold |
| 9-12 | 4 bytes | ADDR_MIN_VAL_HUM | Humidity minimum threshold |
| 13-28 | 16 bytes | ADDR_KEY_FLAG | Device key (12 chars + length) |

# 9. API ENDPOINTS

The system communicates with the following REST API endpoints:

| API Name | Endpoint URL | Purpose |
| --- | --- | --- |
| CO2 API | http://workpanel.in/mash/webservices/sensor_api/sen_co2.php | Send SCD41 sensor data |
| DHT API | http://workpanel.in/mash/webservices/sensor_api/sen_dht11.php | Send DHT11 sensor data |
| Bag API | http://workpanel.in/mash/webservices/sensor_api/sen_ds18b20.php | Send bag temperature data |
| Auth API | http://workpanel.in/mash/webservices/sensor_api/dk_auth.php | Device key authentication |

## 9.1 JSON Data Format

CO2 Sensor Payload:

{"aksi":"sensordata","sensorName":"CO2Sensor","deviceKey":"<12-char-key>", "CO2":"<value>", "Humidity":"<value>", "Temperature":"<value>"}

Bag Sensor Payload:

{"aksi":"sensordata","sensorName":"BagSensor","deviceKey":"<12-char-key>", "bagNumber":"<number>", "Temperature":"<value>"}

DHT Sensor Payload:

{"aksi":"sensordata","sensorName":"DHTSensor","deviceKey":"<12-char-key>", "HumidityOUT":"<value>", "TemperatureOUT":"<value>"}

Authentication Payload:

{"aksi":"device_key","deviceKey":"<12-char-key>"}

# 10. CODE FILE STRUCTURE

The project is organized into multiple Arduino (.ino) files:

| File Name | Description |
| --- | --- |
| main.ino | Main program entry point (setup/loop) |
| configuration.h | Global constants, pin definitions, and object declarations |
| initializeDevices.ino | Device initialization and menu handler |
| bagSensor.ino | DS18B20 temperature sensor reading functions |
| CO2Sensor.ino | SCD41 CO2 sensor initialization and reading |
| dhtSensor.ino | DHT11 sensor reading and heat index calculation |
| joyStick.ino | Joystick input handling and virtual keyboard |
| menuControl.ino | Menu navigation and action execution |
| welcomeScreen.ino | Startup splash screen display |
| getKey.ino | Device key authentication and input functions |
| initWifi.ino | WiFi connection and OTA setup |
| eepromConfig.ino | EEPROM read/write operations |
| relayControl.ino | Relay control and threshold management |
| sendingJsonRequest.ino | HTTP POST request handling for data transmission |

# 11. USAGE INSTRUCTIONS

## 11.1 First-Time Setup

1. Connect all sensors to the correct ports as per the pin diagram

2. Power on the device using a 5V, 2A DC adapter

3. The LCD will display "Connecting to WiFi"

4. If no saved WiFi, a hotspot named "MushroomFarming" will be created (password: "password")

5. Connect your mobile/computer to this hotspot

6. A captive portal will open - enter your WiFi credentials

7. The device will connect and save the credentials

8. Enter the 12-character device key using the virtual keyboard

9. Choose to keep default values or set custom thresholds

## 11.2 Normal Operation

The LCD continuously displays sensor readings:

- Line 1: CO2 level (ppm)

- Line 2: Temperature (°C)

- Line 3: Humidity (%)

- Line 4: Bag temperatures (B1, B2, etc.)

Data is automatically sent to the server every 5 minutes

Device key is re-authenticated every 30 minutes

Relays activate automatically based on sensor thresholds

## 11.3 Accessing the Menu

1. Press the joystick button to enter the menu

2. Use joystick to navigate up/down through options

3. Press button to select an option

4. Menu will timeout after 2 minutes of inactivity

# 12. WIRELESS OTA UPDATES

The system supports Over-The-Air (OTA) firmware updates, allowing remote code deployment without physical access to the device.

## 12.1 OTA Update Procedure

1. Ensure the device is online and connected to WiFi

2. Note the IP address displayed on the LCD screen

3. Export compiled binary from Arduino IDE: Tools > Get Compiled Binary

4. Open a browser and navigate to: http://<device-ip>/

5. Verify "DEVICE ONLINE" message is displayed

6. Navigate to: http://<device-ip>/update

7. The ElegantOTA interface will appear

8. Select the compiled .bin file

9. Click "Update" to upload the firmware

10. Device will automatically restart with new firmware

# 13. TROUBLESHOOTING

## 13.1 Common Issues

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| WiFi not connecting | Wrong credentials/Weak signal | Use Change WiFi menu option to reconfigure |
| Sensor showing "nan" | Sensor disconnected/Faulty | Check wiring and connections |
| Device key invalid | Expired subscription/Wrong key | Contact administrator for valid key |
| LCD not displaying | I2C address mismatch | Check LCD address (0x27 or 0x3F) |
| Relays not working | Wrong pin connections | Verify relay pin wiring |

## 13.2 Reset Codes

Factory Reset Code: AB1234

## 13.3 Default Thresholds

CO2 Minimum: 1200 ppm

Temperature Minimum: 16°C

Humidity Minimum: 90%

## 13.4 Contact Information

For technical support or inquiries:

Developed by: Drift Developers

Client: Organic Court